🧩 INFO1112 — Week 12: Performance Analysis

Lecturer: Dr. Wei Li
Topic: Understanding and evaluating computer system performance
Focus: 延迟 (latency)、吞吐量 (throughput)、排队理论 (queueing theory)、测量陷阱 (measurement pitfalls)

🧠 1. Why Performance Analysis Matters

性能分析的核心目标：

“To understand where time goes and why systems behave slowly.”

Key Points 🔹

- Systems become complex — thousands of interacting components.

- Even small inefficiencies can cause major slowdowns.

- Performance affects usability, scalability, and cost.

📘 In networking & OS, performance = how efficiently resources are used:

- CPU (processing power)

- Memory (allocation, paging)

- Network (bandwidth, latency)

- Disk I/O (storage speed)

⚙️ 2. Key Performance Metrics 关键性能指标

| Metric | 中文 | Description |
| --- | --- | --- |
| Latency | 延迟 | Time to complete one request. Influenced by processing, queueing, transmission, propagation. |
| Throughput | 吞吐量 | Number of requests completed per second. Reflects system capacity. |
| Utilization | 利用率 | Percentage of time a resource is busy. |
| Scalability | 可扩展性 | How performance changes as workload increases. |
| Queueing Delay | 排队延迟 | Time spent waiting for service when system is busy. |

🧩 Latency vs Throughput：

- Latency → 单个任务有多快

- Throughput → 系统整体能并行多少任务

Analogy: a bridge with one car (latency) vs traffic flow per minute (throughput).

🌐 3. Network Delay Components

在网络层面，一个数据包从源到目的地的延迟可以分解为：

dtotal=dproc+dqueue+dtrans+dprop

| Component | 中文 | Meaning |
| --- | --- | --- |
| Processing delay (d_proc) | 处理延迟 | Check bit errors, determine output link (<1ms) |
| Queueing delay (d_queue) | 排队延迟 | Time waiting for transmission (variable) |
| Transmission delay (d_trans) | 传输延迟 | Packet length ÷ link bandwidth (L / R) |
| Propagation delay (d_prop) | 传播延迟 | Distance ÷ propagation speed (d / s) |

📘 Example:
A 1000-byte packet on 1 Mbps link
→ dtrans=(8×1000)/(106)=8ms

📊 4. Queueing Theory (排队理论)

公式：

ρ=LaR

ρ：traffic intensity = system utilization
L：packet length
a：arrival rate (packets/s)
R：link capacity (bps)  每秒钟这条通信链路能传输的 最大比特数

| 状态 | 特征 |
| --- | --- |
| ρ < 0.5 | Stable, low delay |
| ρ ≈ 0.9 | Congested, delay spikes |
| ρ > 1 | Unstable — queue grows infinitely |

M/M/1 Queue 模型

- Average queue length: N=ρ1-ρ

- Average delay: D=1(R/L)-a

(R/L)-a= 服务速率 – 到达速率 = “净处理速率”
→ 表示服务器能“腾出空”的速度。

🧩 As utilization → 1, delay → ∞
即“系统越忙，等得越久”，一种指数级恶化。

🔍 5. Measuring Real Internet Delays

工具：traceroute

- Sends 3 probe packets per hop.

- Measures Round-Trip Time (RTT).

- “*” indicates packet loss or no response.

📘 应用：

- Identify bottleneck routers.

- Analyze cross-continent latency (e.g. Sydney → Singapore → US).

- Validate propagation delay ≈ geographic distance.

🚫 6. Packet Loss & Congestion Control

- 当 router buffer 满 时，新来的分组会被丢弃。

- TCP 会重新传输 (retransmit)，导致延迟上升。

- 高丢包率 ⇒ 吞吐量下降、重传增加、RTT 抖动。

📈 TCP’s congestion control → balance throughput & reliability.

⚡ 7. Throughput (系统吞吐量)

Throughput=min⁡(Rc,Rs,Rbottleneck)

- 由系统中 最慢的链路 (bottleneck link) 决定。

- 例：若 10 条连接共享带宽 R → 每条有效吞吐 ≈ R/10。

- 可用来分析负载均衡 (load balancing) 效率。

📐 8. Performance Metrics Calculation

| Comparison | Formula | Example |
| --- | --- | --- |
| n times faster | SpeedA/SpeedB=n | A = 2× faster than B |
| x% faster | (TBTA)-1 | A = 2s, B = 3s → A is 50% faster |

🧮 9. Averages in Performance Analysis

不同平均方式用于不同情境：

| Type | 中文 | 用途 | 特点 |
| --- | --- | --- | --- |
| Arithmetic Mean | 算术平均 | 简单任务平均 | 易受极值影响 |
| Weighted Mean | 加权平均 | 各任务权重不同 | 更符合真实负载 |
| Geometric Mean | 几何平均 | 对比系统性能 | 不依赖基准系统，但忽略任务差异 |
| Harmonic Mean | 调和平均 | 对速率计算更准确 | 对低速样本敏感，更真实反映整体性能 |

📘 考试重点：

- 几何平均（geometric mean）在 SPEC CPU 基准中常用；

- 调和平均（harmonic mean）适合速率（如吞吐）比较。

⚠️ 10. Pitfalls in Performance Measurement

常见误区：

| 错误类型 | 示例 |
| --- | --- |
| Selective statistics | 只展示最佳结果 |
| Ignoring average behavior | 关注峰值、忽略稳定性 |
| Ambiguous terms | “nearly”, “almost” 造成误导 |
| Mixing metrics | 比较不同维度（CPU vs I/O） |
| Market-driven claims | “Our CPU runs faster” 无具体上下文 |

📘 Real takeaway:

Always question what metric and under what conditions performance is measured.

🧩 11. Benchmarking（基准测试）

| Benchmark | 测试内容 | 应用领域 |
| --- | --- | --- |
| SPEC CPU Standard Performance Evaluation Corporation CPU Benchmark | CPU & memory performance | 通用计算 |
| TPC-C / TPC-H Transaction Processing Performance Council – C / H Benchmark | Database transaction & queries | 商业系统 |
| CloudSuite | Cloud workloads | Web / Analytics |
| MLPerf Machine Learning Performance Benchmark | Machine learning inference | AI 领域 |

📘 Good practice:
Use multiple benchmarks for comprehensive evaluation.

🧠 12. “GHz ≠ Performance”

频率 (GHz) 不是性能唯一指标。
系统性能还取决于：

- 核心数 (cores)

- 缓存层级 (cache hierarchy)

- 指令并行度 (ILP)

- 编译优化

- 程序特征 (I/O-bound vs CPU-bound)

A 3GHz ARM core may outperform a 4GHz x86 core on certain workloads.

🧩 13. Takeaways & Exam-Focused Summary

| 层面 | 概念串联 | 常见题型 |
| --- | --- | --- |
| Network level | 延迟分解 (proc + queue + trans + prop) | 计算 RTT / ρ 值 |
| Queueing theory | ρ=La/R, M/M/1 delay formula | 推导平均延迟 |
| System performance | 几何/调和平均 | 多系统性能比较题 |
| Measurement pitfalls | 如何避免误导统计 | 概念选择题 |
| Benchmarking | SPEC, TPC, MLPerf | 匹配/填空题 |

🧾 Key Formula Summary

| 概念 | 公式 | 说明 |
| --- | --- | --- |
| Total delay | dtotal=dproc+dqueue+dtrans+dprop | 网络端到端延迟 |
| Queue utilization | ρ=La/R | 链路负载率 |
| M/M/1 delay | D=1/((R/L)-a) | 平均等待时间 |
| Throughput | min⁡(Rc,Rs,Rbottleneck) | 系统瓶颈 |
| Speed ratio | SA/SB | n 倍速度比较 |

🎯 Final Thoughts

Performance analysis = balance between efficiency and fairness.
要懂得区分：

- 局部性能（single component）

- 系统性能（end-to-end behavior）

- 理论模型（理想状态） vs 实测性能（现实世界）

“What you measure defines what you optimize.”